This checklist applies to general industry and construction workplaces

Are floor and bench mounted grinders equipped with a work rest that is adjusted to within 1⁄8 inch of the wheel?

Is the adjustable tongue guard on the top side of the grinder used and kept adjusted to within 1⁄4 inch of the wheel?

Is the grinder equipped with flanges?

Do side guards cover the spindle, nut, flange projections, and 75 percent of the wheel diameter?

Does the angular exposure of the grinding wheel periphery and sides for safety guards on your grinders meet the following?

Bench and floor stand grinders – not to exceed 90 ° or ¼ of the periphery

Cylindrical grinders – not to exceed 180°

Surface grinders and cutoff machines–not to exceed 150°

Swing frame grinders – not to exceed 180° with the top half of the wheel enclosed

Automatic snagging machines – not to exceed 180°

Top grinding grinders – not to exceed 60° when work is applied to the wheel above the horizontal centerline.

Are right angle grinders equipped with guards when they are not being used for internal grinding where the material being worked on provides adequate protection?

Are bench and pedestal grinders permanently secured if they are not going to be moved?

Do employees wear ANSI-approved goggles or face shields when grinding?

Is the maximum RPM rating of each abrasive wheel compatible with the RPM rating of the grinder motor?

Does each grinder have an individual on/off switch?

Is each electrically operated grinder effectively grounded?

Before new abrasive wheels are mounted, are they visually inspected and ring tested?

Are dust collectors and powered exhausts provided on grinders used in operations that produce large amounts of dust?

To prevent coolant from splashing workers, are splash guards mounted on grinders that use coolant?

Is cleanliness maintained around grinders?

## Key rules:

- Division 2, Subdivision O, 1910.215, Abrasive wheel machinery

- Division 3, Subdivision I, 1926.300, General requirements, and 1926.303, Abrasive wheels and tools